TCP

Traffic Control Planner

Competitive Analysis & MVP Feature Roadmap

February 2026

Prototype Phase

# Competitive Feature Matrix

Comparison of TCP prototype against RapidPlan (Invarion) and AutoCAD with traffic plugins. Priority ratings indicate importance for our MVP launch.

Priority Key:  Done = Already built  |  Critical = Must have for MVP  |  Important = Competitive parity  |  Nice to have = Post-MVP

| Feature | RapidPlan | AutoCAD+ | TCP (Ours) | MVP Priority |
| --- | --- | --- | --- | --- |
| MAP & LOCATION |  |  |  |  |
| Aerial/satellite imagery | Google, Mapbox, Bing | Import external | OSM, Esri, CartoDB | Done |
| Location search/geocoding | Built-in | Manual | Nominatim | Done |
| Auto-import road layout | Yes - from map data | No | No | Critical |
| Multiple map providers | 3 providers | N/A | 5 providers | Done |
| Map opacity control | Yes | N/A | Yes | Done |
| Geo-referenced coords | Yes | Full coord systems | Yes - lat/lng | Done |
| Dynamic scale bar | Yes | Yes | Yes - feet | Done |
| ROAD TOOLS |  |  |  |  |
| Multi-lane road drawing | Configurable | Full CAD | 2/4/6/hwy | Done |
| Curved/Bezier roads | Yes - splines | Full spline/arc | No - straight only | Critical |
| Intersection editor | Auto intersection creator | Manual | No | Critical |
| Turn lanes / slip lanes | Dedicated tool | Manual | No | Important |
| Road shoulders & sidewalks | Configurable | Manual | No | Important |
| Lane masking/removal | Mask tool | Manual edit | No | Important |
| Crosswalks | Dedicated tool | Blocks | No | Important |
| Train/tram tracks | Rail line tool | Blocks | No | Nice to have |
| Roundabouts | Roundabout tool | Manual/plugin | No | Nice to have |
| SIGNS & DEVICES |  |  |  |  |
| MUTCD sign library | Full state libraries | Via plugins | ~34 basic signs | Critical |
| Searchable sign library | Yes | Plugin dependent | No - browse only | Important |
| Traffic device library | Extensive | Block libraries | 10 devices | Important |
| Sign/device rotation | Yes | Yes | Yes | Done |
| Sign/device scaling | Yes | Yes | Yes | Done |
| Custom sign creator | Convert primitives | Full CAD | No | Nice to have |
| State-specific libraries | Per state/region | Purchased | Generic US only | Important |
| WORK ZONE TOOLS |  |  |  |  |
| Work zone area marking | Yes | Manual | Yes - hatched zones | Done |
| Taper/channelization | Auto-calc from speed | Manual | No | Critical |
| Buffer space indicators | Yes | Manual | No | Important |
| Auto-template (MUTCD) | Typical applications | No | No | Critical |
| Sign spacing auto-calc | Based on speed/class | No | No | Critical |
| Traffic flow arrows | Marked path tool | Manual | Basic arrows | Important |
| ANNOTATION & DOCUMENTATION |  |  |  |  |
| Text labels | Rich text | Full annotation | Basic labels | Done |
| Measurement tool | Geo-accurate | Full dimensioning | Haversine ft + m | Done |
| Auto legend box | Dynamic from objects | Manual | No | Critical |
| Sign/device manifest | Auto-count inventory | Manual | No | Critical |
| Title block / branding | Print frames | Templates | No | Critical |
| North arrow | Yes | Yes | No | Important |
| Notes / callouts | Yes | Yes | No | Important |
| Chainage markers | New in 4.3 | Manual | No | Nice to have |
| EXPORT & PRINTING |  |  |  |  |
| PDF export | High quality | Yes | No | Critical |
| Print to scale | Precise control | Yes | No | Critical |
| Image export (PNG) | Yes | Yes | No | Important |
| CAD export (DXF/DWG) | New in 4.2 | Native | No | Nice to have |
| Multi-page printing | Print regions | Yes | No | Nice to have |
| TEMPLATES & AUTOMATION |  |  |  |  |
| Pre-built plan templates | MUTCD typical apps | Purchased blocks | No | Critical |
| Auto-template tool | Semi-auto from location | No | No | Critical |
| Reusable components | Scratchpad | Blocks | No | Important |
| COLLABORATION & DATA |  |  |  |  |
| Save / load plans | Invarion Cloud | File-based | No - session only | Critical |
| Cloud project library | Map-based browser | Autodesk cloud | No | Important |
| Team collaboration | Live share + comments | Cloud sharing | No | Important |
| Autosave / versioning | 14-day recovery | DWG versioning | Undo/redo only | Important |
| DRAWING TOOLS |  |  |  |  |
| Select / move objects | Yes | Yes | Yes | Done |
| Pan / zoom | Yes | Yes | Yes | Done |
| Undo / redo | Yes | Yes | Yes - full history | Done |
| Copy / paste / duplicate | Yes | Yes | No | Critical |
| Multi-select | Yes | Yes | No - single only | Important |
| Group / ungroup | Yes | Yes | No | Important |
| Object z-ordering | Yes | Yes | No | Important |
| Snap to objects | Yes | Extensive | No | Important |
| Keyboard shortcuts | Yes | Extensive | Yes - 10 tools | Done |

# Current Prototype Strengths

- Web-native: Runs in browser with zero install. RapidPlan requires Windows + .NET desktop app.

- Multi-provider maps: 5 tile sources including satellite, with opacity control.

- Real geo-coordinates: All objects pinned to lat/lng with real-world distance measurement.

- Modern UI: Dark theme, professional design, feels contemporary vs dated competitors.

- Instant start: No account, no download, zero friction to first plan.

- Keyboard shortcuts: 10 tools all accessible via single keypress.

# Competitive Positioning

## vs RapidPlan (Invarion)

We are web-first with zero install, modern UI, and can undercut on price. RapidPlan is Windows-only desktop software with dated UX. Our AI integration potential is a major differentiator they cannot easily replicate.

## vs AutoCAD + Plugins

AutoCAD is a general-purpose CAD tool that is expensive, complex, and has a steep learning curve. We are purpose-built for traffic control, which means faster time-to-plan and lower training costs. AutoCAD requires additional plugins (Autosign, CONE, KeySIGNALS) to achieve what we offer natively.

## Our Moat

Web-native + AI-assisted + purpose-built = fastest path from needing a plan to having a PDF ready for the permit office.

# Phase 1: MVP Features

These features transform the prototype from a demo into a usable tool. Focus: make plans saveable, exportable, and minimally complete for real-world use.

## Estimated Timeline: 1-2 Weeks

| # | Feature | Description & Requirements |
| --- | --- | --- |
| 1 | Save / Load Plans | Save plan state as JSON to localStorage for persistence between sessions. Add JSON export/import buttons for backup and sharing. Plan data includes all objects with geo-coordinates, map center/zoom, title, and metadata. |
| 2 | PDF Export with Title Block | Canvas-to-PDF export using browser print or a library like jsPDF. Include configurable title block with: project name, date, company name, planner name, permit number, and scale indicator. Output should be print-ready at standard paper sizes. |
| 3 | Auto-Generated Legend Box | A dynamic legend that automatically lists all unique sign types and devices currently placed on the plan. Shows icon + label for each. Updates in real-time as objects are added or removed. Can be positioned on canvas. |
| 4 | Sign/Device Manifest | Auto-generated inventory table counting all placed signs and devices by type. Shows: sign code, description, quantity. Useful for procurement and field verification. Should be included in PDF export. |
| 5 | Copy / Paste / Duplicate | Select object(s) and duplicate with Ctrl+C / Ctrl+V or a duplicate button. Pasted objects offset slightly from original. Essential for placing repeated elements like cone lines, delineators, and barrier runs. |
| 6 | Expand MUTCD Sign Library | Increase sign library from ~34 to 80+ signs covering the most common MUTCD categories. Add search/filter functionality to the sign panel. Include sign code numbers (e.g., W20-1, R1-1) for professional use. |
| 7 | Basic Taper Tool | Draw angled taper lines for lane closures. Configurable taper length based on speed limit using MUTCD formula (L = W x S for speeds over 45mph). Visual channelization with cone/barrel spacing indicators. |
| 8 | Image Export (PNG) | Export the current canvas view as a high-resolution PNG image. Useful for quick sharing, embedding in emails, and inserting into other documents. |
| 9 | North Arrow & Compass | A north arrow indicator on the canvas that shows map orientation. Auto-rotates if the map is rotated. Standard element required on all traffic control plans. |
| 10 | Improved Object Properties | Add object-to-object snapping for road endpoints. Bring to front / send to back z-ordering. Multi-select with Shift+click. Basic object alignment tools. |

## Phase 2: Realistic Roads (Weeks 3-5)

Bezier/curved road segments, basic intersection handling, turn lanes, road shoulders, and crosswalk tools. This makes the road drawing competitive with RapidPlan.

## Phase 3: Standards Compliance (Weeks 5-8)

Full MUTCD sign library, taper/channelization with auto-calculation, pre-built MUTCD typical application templates, and sign spacing calculator based on posted speed.

## Phase 4: Collaboration (Weeks 8-11)

User accounts, cloud save, share plans via link, plan commenting, and project library with map-based browser.

## Success Metrics for Phase 1

- A user can create a simple lane closure plan from scratch in under 10 minutes

- The plan can be exported as a print-ready PDF with title block and legend

- Plans persist between browser sessions via localStorage

- The sign manifest accurately counts all placed devices for field verification

- The tool feels faster to start than RapidPlan (no install, no account required)